PFUMA — Hardware Component List

# CN-01 Collar Node

| Ref | Component | Specification |
| --- | --- | --- |
| U1 | ESP32-WROOM-32 DevKit | Dual-core MCU with WiFi and Bluetooth, 38-pin |
| U2 | NEO-6M GPS module | UART, with ceramic patch antenna |
| U3 | SX1278 / Ra-02 LoRa module | 433 MHz, SPI, SMA connector |
| U4 | MPU-6050 IMU | 6-axis accelerometer and gyroscope, I2C 0x68 |
| U5 | MAX30102 | Heart-rate / PPG sensor, I2C 0x57 |
| U6 | DS18B20 waterproof probe | 1-Wire temperature probe, cable with steel tip |
| U7 | TP4056 with protection | LiPo charge controller |
| U8 | AMS1117-3.3 LDO | 3.3 V regulator, SOT-223 |
| U9 | MT3608 boost converter | 3.7 V to 5 V step-up, optional |
| BT1 | LiPo battery | 3.7 V, 2000 mAh, JST-PH connector |
| SP1 | Solar panel | 40 x 70 mm, 5 V, 100 mA |
| ANT1 | LoRa antenna | 433 MHz SMA whip |
| C1-C5 | Capacitors | 100 nF, 10 uF, 100 uF, 22 uF |
| R1-R6 | Resistors | 4.7 kOhm x3, 100 kOhm x2, 330 Ohm |
| LED1 | Status LED | Green, 3 mm |
| SW1 | Tactile button | 6 x 6 mm |
| CN1 | JST-XH connector | 2-pin, solar input |
| CN2 | Female header | 4-pin, 2.54 mm, UART debug |
| PCB | Custom PCB | 80 x 50 mm, 2-layer |
| ENC | Enclosure | 3D-printed, IP54, 80 x 50 x 25 mm |
| STR | Livestock collar strap | 25 mm nylon, adjustable |

# BS-01 Base Station

| Ref | Component | Specification |
| --- | --- | --- |
| U1 | ESP32-WROOM-32 DevKit | MCU with WiFi, 38-pin |
| U2 | SX1278 / Ra-02 LoRa module | 433 MHz, SPI, SMA connector |
| U3 | SSD1306 OLED display | 128 x 64, I2C 0x3C, 0.96 inch |
| U4 | LM2596 buck converter | 12 V to 5 V step-down |
| U5 | AMS1117-3.3 LDO | 3.3 V regulator |
| ANT1 | LoRa antenna | 433 MHz SMA whip |
| C1-C5 | Capacitors | 100 nF x4, 100 uF / 16 V |
| R1-R6 | Resistors | 330 Ohm x4, 4.7 kOhm x2 |
| LED1-4 | Indicator LEDs | Green, blue, yellow, red, 3 mm |
| CN1 | DC barrel jack | 12 V, 5.5 / 2.1 mm, panel mount |
| CN2 | Pin header | 4-pin, 2.54 mm, UART debug |
| PCB | Custom PCB | 2-layer |
| ENC | Project box | ABS, 120 x 80 x 40 mm |
| PWR | DC adapter | 12 V, 1 A wall supply |