PFUMA — Breadboard Assembly Guide

CN-01 collar node and BS-01 base station, wired on solderless breadboards for bench testing before committing to PCB.

# Read first

- Power from USB only while on breadboard. Plug the ESP32 devkit into your PC's USB. Its onboard regulator supplies the 3V3 rail for every module. Do not connect the LiPo battery, solar panel or TP4056 until the design moves to PCB — breadboard contact resistance and a lithium cell are a bad combination, and you do not need battery power to test.

- Everything runs at 3.3 V. Take module VCC from the 3V3 rail, never from the devkit's 5V/VIN pin. The SX1278 in particular is not 5 V tolerant and will be damaged.

- Fit the LoRa antenna before powering up. Transmitting without an antenna can destroy the SX1278 output stage.

- Build in stages. Each stage below ends with a check. Power down before adding the next module, and do not move on until the current stage passes.

- Use the female header strips. Solder them to any module that arrived without pins, so modules push into the breadboard and can be pulled out again.

# Known correction to the design document

hardware/HARDWARE_DESIGN.md section 2.3 assigns GPIO5 to both the yellow WiFi LED and the LoRa chip-select on BS-01, and carries a stale note about reassigning it. The firmware resolved the clash and the document was never updated. Wire BS-01 to the values below, which match base_station.ino:

| Signal | Correct pin |
| --- | --- |
| LoRa NSS | GPIO5 (unchanged) |
| WiFi LED, yellow | GPIO15 (not GPIO5) |
| Alert LED, red | GPIO13 (not GPIO15) |

CN-01's netlist and firmware agree, so no correction is needed there.

# Breadboard 1 — CN-01 Collar Node

Six stages, 28 connections. Seat the ESP32 devkit across the centre channel at one end of the board so both pin rows are reachable, leaving roughly two thirds of the board free for sensors.

## Stage 1 — Power rails

| From | To |
| --- | --- |
| ESP32 3V3 | Red (+) rail, both sides |
| ESP32 GND | Blue (-) rail, both sides |
| Red rail left | Red rail right |
| Blue rail left | Blue rail right |

Check: USB in, the devkit's own power LED lights. Meter across the rails reads 3.3 V.

## Stage 2 — SX1278 LoRa radio

| From | To |
| --- | --- |
| SX1278 3.3V | 3V3 rail |
| SX1278 GND | GND rail |
| SX1278 MISO | ESP32 GPIO19 |
| SX1278 MOSI | ESP32 GPIO23 |
| SX1278 SCK | ESP32 GPIO18 |
| SX1278 NSS | ESP32 GPIO5 |
| SX1278 RST | ESP32 GPIO14 |
| SX1278 DIO0 | ESP32 GPIO26 |
| SX1278 DIO1 | ESP32 GPIO12 |
| SX1278 DIO2 | ESP32 GPIO13 |

Screw the 433 MHz whip onto the SMA connector now. Check: the firmware's LoRa init succeeds on the serial monitor at 115200 baud.

## Stage 3 — I2C sensors

MPU-6050 and MAX30102 share one bus. Run two jumpers from the ESP32 to spare breadboard rows, then branch both modules off those rows.

| From | To |
| --- | --- |
| ESP32 GPIO21 | SDA row |
| ESP32 GPIO22 | SCL row |
| MPU-6050 VCC | 3V3 rail |
| MPU-6050 GND | GND rail |
| MPU-6050 SDA | SDA row |
| MPU-6050 SCL | SCL row |
| MPU-6050 AD0 | GND rail (sets address 0x68) |
| MPU-6050 INT | ESP32 GPIO35 |
| MAX30102 VIN | 3V3 rail |
| MAX30102 GND | GND rail |
| MAX30102 SDA | SDA row |
| MAX30102 SCL | SCL row |

Both GY-521 and MAX30102 breakouts carry their own I2C pull-ups, so the discrete 4.7k resistors R5 and R6 are for the PCB, not the breadboard. Check: an I2C scanner sketch reports 0x68 and 0x57.

## Stage 4 — DS18B20 temperature probe

| From | To |
| --- | --- |
| DS18B20 red wire | 3V3 rail |
| DS18B20 black wire | GND rail |
| DS18B20 yellow wire | ESP32 GPIO4 |
| 4.7k resistor | GPIO4 row to 3V3 rail |

The 4.7k pull-up is required — the probe has no onboard resistor and will read -127 without it. Check: a plausible room temperature on serial.

## Stage 5 — NEO-6M GPS

| From | To |
| --- | --- |
| GPS VCC | 3V3 rail |
| GPS GND | GND rail |
| GPS TX | ESP32 GPIO16 |
| GPS RX | ESP32 GPIO17 |

TX goes to GPIO16 and RX to GPIO17 — crossed, not straight through. Check: NMEA sentences on serial. A first fix needs a clear sky view and can take several minutes.

## Stage 6 — Status LED and button

| From | To |
| --- | --- |
| ESP32 GPIO2 | 330 ohm resistor |
| 330 ohm resistor | LED anode, long leg |
| LED cathode, short leg | GND rail |
| Button leg 1 | ESP32 GPIO0 |
| Button leg 2 | GND rail |

Check: the LED follows the firmware's heartbeat.

# Breadboard 2 — BS-01 Base Station

Four stages, 18 connections. Simpler than the collar: no GPS, no sensors, and it runs on mains rather than a battery.

## Stage 1 — Power rails

| From | To |
| --- | --- |
| ESP32 3V3 | Red (+) rail, both sides |
| ESP32 GND | Blue (-) rail, both sides |
| Red rail left | Red rail right |
| Blue rail left | Blue rail right |

Leave the LM2596 and the 12 V adapter out of the breadboard build. They exist to run the finished box from mains; USB is sufficient on the bench.

## Stage 2 — SX1278 LoRa radio

Identical to CN-01 stage 2, except DIO1 and DIO2 are not used — the base station firmware only reads DIO0.

| From | To |
| --- | --- |
| SX1278 3.3V | 3V3 rail |
| SX1278 GND | GND rail |
| SX1278 MISO | ESP32 GPIO19 |
| SX1278 MOSI | ESP32 GPIO23 |
| SX1278 SCK | ESP32 GPIO18 |
| SX1278 NSS | ESP32 GPIO5 |
| SX1278 RST | ESP32 GPIO14 |
| SX1278 DIO0 | ESP32 GPIO26 |

Both radios must be on the same frequency, spreading factor and bandwidth: 433 MHz, SF7, 125 kHz. These are already set in both .ino files.

## Stage 3 — SSD1306 OLED

| From | To |
| --- | --- |
| OLED VCC | 3V3 rail |
| OLED GND | GND rail |
| OLED SDA | ESP32 GPIO21 |
| OLED SCL | ESP32 GPIO22 |

Check: an I2C scanner reports 0x3C, then the firmware's splash screen appears.

## Stage 4 — Indicator LEDs

Four LEDs, each through its own 330 ohm resistor to the GND rail. These are the corrected pins.

| From | To |
| --- | --- |
| ESP32 GPIO2 | Green LED, power |
| ESP32 GPIO4 | Blue LED, LoRa activity |
| ESP32 GPIO15 | Yellow LED, WiFi |
| ESP32 GPIO13 | Red LED, alert |

Long leg to the resistor, short leg to ground, on every one.

# End-to-end check

With both breadboards powered and flashed:

- Set the WiFi SSID, password and API URL in the configuration block of base_station.ino before flashing it.

- Watch the collar's serial output for a transmit line, and the base station's for a matching receive with an RSSI figure.

- Keep the two boards at least a couple of metres apart. Touching antennas can saturate the receiver and look like a failure.

- Pair both device serials in the PFUMA app before expecting readings to be stored — the telemetry endpoint rejects unrecognised serials.